# Carnegie Mellon University

Department of Civil and Environmental Engineering

12-725:  Fate, Transport, and Physicochemical Processes of Organic Contaminants in Aquatic Systems and their Measurement in Environmental Samples

Problem Set #6 Spring 2020-SOLUTIONS

Due March 5, 2020 by 5pm

- Describe the general chemical and structural features of “organic matter” in the environment. What are the primary sources of organic matter in the environment? What are the main differences between soil organic matter (SOM), particulate organic matter (POM), and dissolved organic matter (DOM)? What are the primary thermodynamic driving forces for sorption of organic compounds to organic matter from air and water?

Organic matter is the environment is made up primarily of C, O, H, N, with some P and S. It has all of the typical features of plant and animal organic molecules including aromatics, OH groups, COOH groups, and some phosphate and S groups. Some organic matter is derived from combustion. It is primarily carbon, with limited OH and COOH functionality. Sources of OM are varied, but include decayed plant matter, combustion products, and waste and decay products from animals.

SOM, POM, and DOM are different types of organic matter. SOM is typically humic materials from partially decayed plant matter (e.g. leaf litter). It is poorly water soluble and can be relatively large in size. Particulate organic matter is a very similar to SOM, and can be considered a subset of SOM. It may be slightly more oxidized and decomposed than SOM, comprised on humic acids and fulvic acids. Dissolved organic matter is the most soluble and lowest MW fraction of organic matter. It is most oxidized (OH and COOH groups), more fulvic acid like than humic acid like, and passes through a 0.45 micron filter.

Organic compounds (neutral) adsorb to OM based on their hydrophobic interaction with SOM, POC, or DOM. It allows them to minimize contact with water and increase contact with hydrophobic surfaces.

- Q13.2 and Q13.7 in EOC
Q13.2 Besides NOM, black carbon can play a significant role in sorption in soils and sediments. It is also possible that wood, plant debris, micro plastics, and other hydrophobic materials may act as sorbent, although the affinity for these surfaces tends to be lower than for NOM.

In aquifers (e.g. carbonate rocks or sandstones), the mineral surfaces may also serve as sorbents.  This is of grater impact for organics with functional groups that may be charged or form specific interactions with mineral surfaces.

In surface waters, all of these other solids may be important, as well as living plants and organisms.

Any surface where “adsorption” occurs (e.g. black carbon, mineral surfaces) can lead to non-linear partitioning.

Q13.7 Neutral organic adsorb to NOM in a pH-independent manner. The sorption is driven primarily by hydrophobic interactions as described above. Charged species will interact differently with NOM due to electrostatic repulsions and/or specific interactions. These charged species are more pH dependent that neutral species.

An organic acid will adsorb more at lower pH where the compound tends to be protonated and uncharged. Similar pH-dependent behaviors can occur for organic bases that can he positively charged at lower pH (protonated amine groups) giving rise to greater adsorption at lower pH compared to higher pH (e.g. see Fig. 13-9).

- P13.2 and 13.7 in EOC

P13.2

5 pt for each correct value

- 4 pt for right formula

- 1 pt for right answer

In order to first get a rough estimate of the fraction present in dissolved form in the three systems, take an average Kiocw value of about 100 L kgoc–1. Note that if a more accurate value would be needed you could still look at scenarios using different Kiocw values. Assuming that Kid = focKiocw , the fraction in dissolved form is given as (Eq. 12.18):

fiw 11rswKid11rsw focKiocw

(a)(b) the solid-water ratio, rsw, is about equal to the particle concentration resulting in:

(a) fiw = 0.9998

(b) fiw = 0.998

(c) In the aquifer where the solid-water ratio is much larger, it is given by (Eq. 12-22):

rsw ρs1-φφ

Hence, rsw = 10 kg solid L–1 water and therefore, fiw = 0.167, which would mean a retardation factor Rfi

(Eq. 12-25) = (fiw)–1 = 6.

Note that in cases (a) and (b) a more accurate knowledge of Kiocw would not make a big difference for assessing the partitioning behavior of atrazine in the two surface waters. In contrast, in case (c) differences in Kiocw have a more significant effect. Inspection of Fig. 13.2 shows that most of the reported Kiocw values are between 80 and 250 L kg–1 oc. This means that, in the aquifer considered, the predicted fiw values vary between 0.2 and 0.071 and thus the retardation factors between 5 and 14.

P13.7

Correct and similar explanations 15pt

- when 	Cis = 1 micrograms/L, BC has strong affinity.

Chelsea-I:

Insertion of foc, Kioc, fbc, and Kibc into Eq. 13-24 yields Cis values for Ciw=1 and 100 μg L-1, respectively: for Ciw = 1 μg L-1:

Cis = (0.056)(104.3)(1) + (0.00056 to 0.0056)(105.9)(1)0.7 = 1100 + (440 to 4400) = 1540 to 5500 μg kg-1 solid (observed 3200 μg kg-1 solid)

Note that a calculation based only on the product, focKioc would underestimate the observed values by about a factor of three.

for Ciw = 100 μg L-1:

Cis = (0.056)(104.3)(100) + (0.00056 to 0.0056)(105.9)(100)0.7

= 110,000 + (11,000 to 110,000) = 121,000 to 220,000 μg kg-1 solid

(observed 91,000 μg kg-1 solid)

In this case, the estimate based only on focKioc is very close to the experimental result, indicating that for

high substrate concentrations partitioning into POM is the dominant sorption mechanism.

For EPA-23 lake sediment, foc was measured as 0.026 kg oc kg-1 solid. Again assuming the same Kiocw and

Kibcw values for phenanthrene and the same fbc range, one obtains:

for Ciw = 1 μg L-1:

Cis = (0.026)(104.3)(1) + (0.00026 to 0.0026)(105.9)(1)0.7

Cis = 520 + (210 to 2100) = 730 to 2600 μg kg-1 solid (observed 1700 μg kg-1 solid)

and

for Ciw = 100 μg L-1 :

Cis = (0.026)(104.3)(100) + (0.00026 to 0.0026)(105.9)(100)0.7

Cis = 52,000 + (5200 + 52,000) = 57,000 to 104,000 μg kg-1 solid (observed 51,000 μg kg-1 solid)

Hence, as for the Chelsea I top soil, focKiocw underestimates the observed sorption by about a factor of

three at Ciw = 1 μg L-1, whereas at 100 μg L-1, sorption appears to be dominated by POM-water

partitioning.

- A) Consider an environment consisting of air, water, soil, sediment, suspended aquatic matter, and biota.  Beginning with a system mass balance, derive general expressions for the fraction of mass of a species in each phase in terms of the dimensionless partition coefficients, Ki,x, between phases (e.g. Henry’s Constant, KH’) and the phase volumes (Va, Vw, Vsoil, Vsed, etc.).

B) Using your equations (model), calculate the partitioning (i.e. fraction of the compound in each phase) of DDE in an environment with the following properties:

| Compartment Properties |  |  |  |
| --- | --- | --- | --- |
|  | Volume | Density | Organic Fraction |
|  | [m3] | [kg/m3] | [--] |
| Air | 2.00E+09 | 1.19E+00 | N/A |
| Water | 9.90E+05 | 1.00E+03 | N/A |
| Soil | 1.40E+04 | 1.50E+03 | 2.00E-02 |
| Sediment | 9.90E+03 | 1.50E+03 | 4.00E-02 |
| Susp. aq matr | 3.50E+01 | 1.50E+03 | 4.00E-02 |
| Biota | 3.50E+00 | 1.00E+03 | 24% lipid |

5.	P16.2 in EOC

a)

- 5 pt for all correct values

- 5 pt for temperature correction

- 5 pt for comment

Estimate the triolene-water partition coefficents using the given LFER.

Log Kitrw=1.031logKow+0.11

BisphenolA (BPA), Log KBPA,trw=1.031(3.4)+0.11=3.6

Naphthalene, Log KNP,trw=1.031(3.38)+0.11=3.6

Using values from Table 16.2 for storage lipids and the problem statement

LogKislw=vslwV+lslwL+sslwS+aslwA+bslwB+c

For BPA, LogKislw=1.99(1.86)+0.58(9.60)+(-1.62)(1.56)+(-1.93)(0.99)+(-4.15)(0.91)+0.55=1.6

For Naphthalene, LogKislw=1.99(1.09)+0.58(5.16)+(-1.62)(0.92)+(-1.93)(0)+(-4.15)(0.2)+0.55=3.4

Table 16.2 is for 37°C, need to convert to 25°C. Assuming ∆H=-25 kJ/mol

lnK37°CK25°C=-∆HR1298-1310=0.39

Thus, for BPA: Ki tr w=101.6×e0.39=58.8 L/kg

For Naphthalene, Ki tr w=103.4×e0.39=3710 L/kg

The two methods agree well for naphthalene. This makes sense because the training set used these types of aplolar compounds. Thus, a simple single parameter LFER works OK.  However, the two methods do not agree for the more polar compound BPA. This is because the training set does not include compounds with polar groups like those found on BPA. So, even though the Kow values are similar for BPA and naphthalene, the single parameter LFER fails. The multi-parameter LFER is better for BPA.

b) need this

- 5 pt for good assumption (lipid)

- 5 pt for calculation of Kdlip or correct reasoning

- 5 pt for not good assumption

For apolar and weakly polar compounds like naphthalene, logKiow=3.4

For apolar compounds in lipid:  logKi lipw=0.91logKiow+0.50=3.59

Ki lipw=3.9×103L/kg

For proteins: logKi protein w=0.7Kiow=2.38

Ki protein w=240

Since lipid takes 12.9% of dry weight:

Kd lip=0.129×3900=503.1 L/kg

The Kd from protein in:

Kd protein=0.602×240=144.5 L/kg

The Kd from lipid is higher than protein, thus it is reasonable to assume naphthalene partitioned primarily to lipid. Also the Kd of lipid (503.1 L/kg) is close to the measured BAF (525 L/kg), which further confirmed this assumption.

For BPA, the partition coefficient to lipid is approximated by the partition coefficient to triolein due to their similar structures: logKi trw=1.99(1.86)+0.58(9.60)+(-1.62)(1.56)+(-1.93)(0.99)+(-4.15)(0.91)+0.55=1.6

Ki lipw=Ki tr w=39.8

Kd lip=0.129×39.8=5.13

The affinity of bipolar BPA to lipid is much lower compared with apolar compounds like naphthalene. Also, the proteins and carbohydrates are abundant in bipolar groups like hydroxyl groups and amine groups, which can interact strongly with BPA through H-bonding. So, the partition of BPA to protein and carbohydrates can be more important than lipid.